- Write MIPS assembly code in your text editor then save it into project root folder for example:

beq $zero, $zero, MAIN

FUNCTION: addi $s0, $zero, 125

END_FUNCTION: jr $ra

MAIN: jal 1

addi $t0, $zero, 0

addi $t1, $zero, 10

FOR: slt $t2, $t0, $t1

beq $t2, $zero, END

lw $s0, 0($t0)

addi $t0, $t0, 1

beq $zero, $zero, FOR

END: sll $zero, $zero, 0

EXIT_MAIN:

- Open terminal in project’s folder then run assembler by running this  command (given that you named the code file “code.mips”)

java –jar assemble.jar code.mips

Note: You can alternatively run “assemble ‘code.mips’” file.

- Write data memory contents in “data-memory.mem” in binary one byte in a line for example

00000000

00000001

00000010

00000011

00000100

00000101

00000110

00000111

00001000

00001001

00001010

00001011

00001100

00001101

- Open project in ModelSim.

- Compile source code by selecting “Compile All” from “Compile” menu.

- Start simulation by selecting “Start Simulation...” from “Simulate” menu. From work directory choose ‘MIPS’

- If wave form view is not displayed check ‘Wave’ from ‘View’ menu. Select all the signals that you want to trace from ‘Objects’ view then drag them to ‘Wave’ view

- Set simulation run step to 1000 ns then click run

- Continuously click run to simulate next clock cycle